OPENING YOUR POOL

- CHLORINE TABLETS- Keep 2-4 tablets (depending on pool size) in pool at ALL TIMES!

- LIQUID SHOCK- One gallon a week to keep it clear.

- PROTEAM SUPREME (better than algaecide)- recommended to use 3 time a year (1 time when opening, middle of July, and when you close it)

- PH & Alkalinity Up - Use your test strips to test your water and add PH Up or Alkalinity Up as needed according to the chart on the back of the chemicals bag.

*Clarifier may be necessary if pool is cloudy. This is the last step you want to take. Clarifier will flock small particles together and drop them to the bottom.

1) Pour in pool (use amount directed on bottle)

2) Let filter run 3-4 hours

3) Turn filter off overnight

4) Sweep

# CHANGE YOUR SAND EACH YEAR

- Empty all old sand from filter tank. Fill filter tank ½ full of water.

- Put tape or cap over the center pipe.

- Slowly fill sand into filter tank.

- Remove tape cap from the center pipe.

- Place top valve back on.

## BACKWASH THE FILTER BEFORE FILTERING

- Turn pump off.

- Turn valve to backwash position.

- Rinse on backwash position for 30 seconds.

- Turn pump off.

- Turn valve to rinse position for 30 seconds

- Rinse on rinse position for 30 seconds.

- Turn pump off.

- Turn pump back to the filter position.

- Turn back on.

NOTICE: We know you should buy your chemicals at ONE PLACE ONLY, especially if you own an automatic feeder. Chemicals have different ingredients and gases. If you mix chemicals they could blow up.